Introduction: Exploring Brain Anatomy

This week’s exercises will let you explore the anatomical organization of the brain to learn some of its basic properties, as well as the location of different structures.

- The human brain is the most complex object in the known universe. Although small, weighing only about 3 pounds (1.3kg) in the adult human, the brain contains approximately 100 billion neurons and perhaps several times as many supporting cells.

- While understanding the basic properties of these neurons and their connections is critical for neuroscience, MRI operates on too coarse of a spatial scale to study neurons directly. Instead, MRI investigates the gross anatomy of the brain, examining structure at the level of millimeters. For comparison, neurons are on the order of a few tens of micrometers in size.

- In this laboratory, you will learn how to use SPM to view anatomical and functional MR images, how to view statistical results of a functional contrast, and to identify some major anatomical landmarks.

Software

Please start by creating the following folder on your laptop: Neurosci382

In that folder create another folder called: Software

Unzip the SPM file that you've downloaded and move spm12 folder into the Software folder.

Then start Matlab and click on "Set Path" in the Home tab. Click "Add Folder..." and add the spm12 folder. Then click on "Save" to save this path structure.

You should be good to go now.

Data Sets Used

For this lab, we will be using a dataset that has already been acquired. To download the dataset that we are going to look at today, go to the Canvas page for the class, and look under Files/Week 2, where you will find Lab1_Data.zip.

Once you've downloaded the data, create a folder called Lab1 in the Neurosci382 folder that you've just created, then copy and paste the unzipped data into the Lab1 folder. This folder is where you will be working on the data.

This exercise uses one anatomical (MRI) data set (in the folder Anat) and one functional (fMRI) data set (in the folder Func).

The Anat dataset contains a single high-resolution anatomical image of the brain of a healthy young adult. The image was collected using an inversion-prepared 3D spoiled-grass (SPGR) pulse sequence, which provides excellent T1 contrast (see Chapters 3 & 4 for more information on T1 pulse sequences). The image dimensions are 256 (X) * 256 (Y) * 162  (Z), with a voxel size of 0.938mm * 0.938mm * 1mm. That is, each slice has 256x256 pixels, and there are 162 slices.

The Func data set contains a time series of images of the same brain as Anat, collected using a gradient-echo spiral sequence, which provides BOLD contrast. The image dimensions are 64 (X) * 64 (Y) * 34 (Z) * 160 (T), with voxel size of 3.75mm * 3.75mm * 4mm. That is, each slice has 64x64 pixels, and there are 34 slices per volume. There are 160 separate time points in each series, collected at a repetition time (TR) of 2s. Note that although this data set is of the same subject’s brain as Anat, it has a different voxel size from Anat and thus the XYZ coordinates do not correspond. The Func directory contains data collected using a blocked (20 seconds each, or 10 images) finger-tapping task of [Rest  -  Right hand finger tapping  -  Rest  -  Left hand finger tapping]; this sequence was repeated four times.

Viewing the brain

You should understand how to load and view two types of data, anatomical images and functional images. Anatomical images are high-resolution images that provide information about the organization of brain anatomy. They are usually sensitive to a tissue contrast parameter like T1 or T2, or to the density of protons of a tissue type. Functional images are usually of lower resolution, but provide information about the changes in brain function over time. Most functional images generated by fMRI experiments are sensitive to blood-oxygenation-level-dependent (BOLD) contrast.

There are several freely distributable or commercial programs available for viewing brain images collected from an MRI scan. In this class, we’ll mostly be using Statistical Parametric Mapping (SPM):

SPM - powerful analysis software for fMRI data:

http://www.fil.ion.ucl.ac.uk/spm/

Another software that lets you view MRI and fMRI data is MRIcron; it is mainly for viewing, not for analyzing data, and it doesn’t require Matlab:

MRIcron - standalone software with viewing utilities for MRI images:

http://www.cabiatl.com/mricro/

TO VIEW THE BRAIN

- In Matlab, click on “Browse for Folder” and select the Neurosci382/Lab1 folder.

- Type spm fmri at the Matlab command window prompt (this will start SPM)

- Then click on the Display button in the upper left window and navigate to the Anat folder in the Neurosci382/Lab 1/ folder.

- Select the file bia5_20105_003.nii and then click Done to open it

- You should now see the image appear in 3 orthogonal planes in the tall window to the right.

** At this point, you should begin working with your partner. **

** I will be available for help as needed. **

1. Using SPM to look at anatomy

The image is in the  .nii, or NIfTI (Neuroimaging Informatics Technology Initiative) format, which is a universal image format for MRI data.

SPM displays 3-dimensional data in each of the 3 orthogonal planes of view (Coronal, Sagittal, Axial, clockwise from the left). Click and drag through each of the views to see how the other 2 planes react. The current 3D coordinate of the cursor is displayed in the top left panel below the images in both voxel space (vx), and millimeter space (mm).

You can increase the brightness of the display by going to SPM Figure > Colours > Effects > Brighten a couple times, if the brain appears a bit dark.

QUESTIONS:

Q1:  While viewing the Anat data set, quickly sketch axial/horizontal slice z=90 by entering [130 130 90] in the right Window in the "Crosshair Position" field under "vx:".

2. Using SPM to look at functional images

Open the functional MR image: Select "Display", navigate to the Neurosci382/Lab 1/Func folder, select bia5_20105_006_01.nii and then click Done to open it.

Q2: While viewing the Func data set, quickly sketch axial slice z=14 (e.g. at "vx:" coordinate [32 30 14]).

Q3:  The two slices (in the anatomical and functional images) are from approximately the same location within the brain. What differences are apparent in the visible anatomy?

[To view both simultaneously, select "Check Reg" in the Menu window, and then select bia5_20105_003.nii in the Anat folder and then bia5_20105_006_01.nii in the Func folder.]

Open the functional MR image again via "Display". Now right click on any of the panes and choose the "Browse option". You will be prompted to select an image; click on the currently selected file (it is highlighted) to remove it, and then enter the string   "^bia5.*" in the field right of "Filter", and "1:160" in the "Frames" field just below. In the field above that now shows multiple files, right click and choose "Select All". Click "Done".

You will now see a horizontal scrolling bar at the bottom of the Graphics window. Clicking on the right or left arrows will advance or go back one volume; you can also click and drag the scrolling bar to view the volumes more rapidly. Clicking on the ">" button (to the right of the right arrow) in the bottom will start movie mode, which flips through the volumes at a rapid pace. Clicking on the button again will stop the movie.

Notice how the brain moves ever so slightly, reflecting how the subject's head moved over the duration of the scan (160 volumes @ 2s each = 320 s = 5.33 mins).

To see how the BOLD signal changes over time in a given voxel, right click on any of the three views, select "Browse..." and then select "Display Profile". This opens up another figure that you can move to the side and thereby view simultaneously as you click on different parts of the brain (the new Figure will show the BOLD signal time course at each voxel that you've clicked on).

Questions:

Q4:  Go to the voxel with coordinates [42,35,24] (see above). This is a voxel in left motor cortex; sketch the time course of its activation and note how the signal intensity (y-axis) varies over time. Then, click on a random voxel in the Visual cortex and sketch its activation; note how it has a noticeably different pattern (one that's random and doesn't show the marked ups and downs as the voxel in motor cortex).

Q5: SPM's Display option provides 3 orthogonal views simultaneously. Why might brain researchers prefer one view of the brain or another?

3. Viewing Functional Activations

Finally, we can view areas in the brain that show statistically significant variations in their BOLD signal as a function of the task (finger tapping).

To do this, click on "Results" in the Menu window. Navigate to the Func folder, select the file "SPM.mat" and then click "Done".

This opens up the "SPM contrast manager". Click on "Effects of Interest" and then on "Done". In the Results window in the bottom left, choose "none", then "FEW", and then press Enter to confirm the p-value threshold of 0.05, and then again to confirm the voxel extent threshold of 0.

The Graphics window will now show the results in graphical form, superimposed on a 'glass brain'. In this contrast, all voxels that showed any kind of significant variation with the task (positive or negative) are highlighted.

To view with more precision where these significant activations are, in the Results window click on "overlays..." and then select "sections". Navigate to the Neurosci382/Lab 1/Anat folder and select wmbia5_20105_003.nii, and then click "Done". In the bottom of the Graphics window, the significant activations are now superimposed onto the high-resolution anatomical (MRI) scan of the subject. Scroll/Click through the brain to inspect the significant activations.

Go ahead and look at other functional contrasts (aka 'statistical comparisons') by selecting "Contrasts" in the Results window, then "Change Contrast", and then, for example, select "Right Hand  >  Left Hand". This contrast shows areas in the brain which had significantly stronger neural activation (BOLD signal) while the subject was tapping their right hand compared to when they were tapping their left hand.

To see these activations superimposed on the subject's anatomical scan again, click on "overlays..." and then select "previous sections".

4. Identifying Anatomical Locations

In the final exercise you will identify select anatomical locations within the structural MRI image in the Anat folder. When running fMRI studies, it is important to gain a working knowledge of brain anatomy, for several reasons. Researchers should be able to accurately and consistently identify areas of activity, to best develop and test brain-based hypotheses. Without a good knowledge of anatomy, it is difficult to read the neuroimaging literature or to converse with other investigators about experimental data. Nevertheless, learning neuroanatomy is hardly a simple task.

The specific anatomical locations are listed as Q6 at the end of these exercises.

To identify brain regions using SPM, load the anatomical image wmbia5_*.nii in the Anat folder (see above) using "Display" and then scroll/click through the slices as needed and click on the desired location to get its voxel coordinates in the "mm:" field. You might also want to brighten the display a couple times to view them better: SPM Figure > Colours > Effects > Brighten

Suggested online atlases:

  - http://www9.biostr.washington.edu/da.html

  - http://www.med.harvard.edu/AANLIB/home.html

Names:____________________

Summary of Exercises

Q1:  While viewing the Anat data set, quickly sketch axial/horizontal slice z=90 by entering [130 130 90] in the right Window in the "Crosshair Position" field under "vx:".

Q2:  While viewing the Func data set, quickly sketch axial slice z=14, i.e. Orig(x,y,14).

Q3:  The two slices (in the anatomical and functional images) are from approximately the same location within the brain. What differences are apparent in the visible anatomy?

Q4:  Go to the voxel with "vx:" coordinates [42,35,24] (see above). This is a voxel in left motor cortex; sketch the time course of its activation and note how the signal intensity (y-axis) varies over time. Then, click on a random voxel in the Visual cortex and sketch its activation; note how it has a noticeably different pattern (one that's random and doesn't show the marked ups and downs as the voxel in motor cortex).

Q5: SPM's Display option provides 3 orthogonal views simultaneously. Why might brain researchers prefer one view of the brain or another?

Q6: Open the Anat/bia5_20105_003.nii structural scan via Display. Choose 5 of the anatomical structures listed below and identify the [X Y Z] voxel coordinate in mm [sic!] near the structure's center of mass (for bilateral structures, chose the right side):

Approximate location of the:

… Amygdala:						X _24__ Y __24__  Z _-16_

… Anterior Commissure:				X _____ Y_____   Z_____

… Posterior Commissure:				X _____ Y _____  Z _____

… Pons:						X _____ Y _____  Z _____

… Inferior Frontal Gyrus:				X _____ Y _____  Z _____

… Anterior Cingulate Gyrus:				X _____ Y _____  Z _____

… Hippocampus:					X _____ Y _____  Z _____

… Central Sulcus:					X _____ Y _____  Z _____

… Inferior Colliculus:					X _____ Y _____  Z _____

… Insular cortex:					X _____ Y _____  Z _____

… First transverse gyrus of Heschl:			X _____ Y _____  Z _____