Understanding k-Space

This week's exercises will illustrate the principles of k-space, and how k-space images relate to real images.

Remember from the text and lecture that raw MRI data are collected as images in k-space, which then must be converted (or “reconstructed”) into real images. It is important to understand the key concepts of k-space for several reasons:

- First, the pulse sequences used for functional MRI differ in their sampling patterns in k-space. Echo-planar sequences traverse k-space in a line-by-line fashion, while spiral sequences adopt a whirlpool (i.e., spiral) motion. These different sampling patterns result in different advantages, disadvantages, and artifacts.

- Second, and most importantly, the k-space representation of the data is integral to the signal reception process for MRI. Understanding k-space will provide a link between the basic physics principles underlying the generation of MR signal and the different pulse sequences used to create images.

Datasets Used

This exercise uses the same anatomical dataset (Anat) we used in the previous lab.

The Anat dataset contains a single high-resolution anatomical image of the brain of a healthy young adult. The image was collected using a inversion-prepared 3D spoiled-grass (SPGR) pulse sequence, which provides excellent T1 contrast (see Chapters 3 & 4 for more information on T1 pulse sequences). The image dimensions are 256 (X) * 256 (Y) * 162  (Z), with a voxel size of 0.938mm * 0.938mm * 1mm.

1. Loading the Data

We will be using Matlab for these exercises. To load the data, in Matlab first navigate to the Anat directory in which the data is stored, then type the following commands into the Matlab command window, pressing Enter after each:

V = spm_vol('bia5_20105_003.nii');

anat.data = spm_read_vols(V);

This will put the entire anatomical image in a Matlab structure called anat. If you type

size(anat.data)

in the command window, you will see that it is a 256x256x162 3D matrix. The third dimension is the slice index (the anatomical image actually consists of a series of 162 horizontal slices along the z-direction), and each slice is a 256x256 2D matrix (corresponding to the phase and frequency gradients). We will now extract a single slice (slice 80, in this case) from this data to work with:

anatslice80=anat.data(:,:,80);

You can look at this slice with:

imagesc(anatslice80);

colormap('gray')

axis('image')

2. (Re)Creating and viewing k-space data

To see what this image looks like in k-space, we must take the 2D Fourier transform:

kanatslice80=fftshift(fft2(anatslice80));

fftshift is a command which will move the center of k-space into the center of the Matlab matrix, fft2 performs the 2D Fourier transform. We can now look at the k-space data:

figure    			% creates a new figure window

imagesc(abs(kanatslice80));  	% the results from the Fourier Transform are complex valued, we just want to see the magnitude (i.e. abs)

colormap('gray')

axis('image')

In this image, you can mostly just see a bright spot in the middle, since the magnitude of the low frequencies is so high, which makes everything else appear dark. We can make this a little easier to see by showing the magnitude values on a log scale:

imagesc(abs(20*log10(kanatslice80)));

You should now be able to see more clearly what the raw k-space data look like. Let's plot the absolute magnitude values in row 129 of k-space (this row runs through the center of k-space):

figure

plot(abs(kanatslice80(129,:)));

You can see the large magnitude at the center (low frequency). Try plotting a different line in k-space to see how the magnitude values vary across k-space.

3. Effect of changing a value in k-space

To understand k-space even better, we will now look at what happens if a single point in k-space is changed. This can happen when there is a problem in the electronics used in acquiring the data (aka a 'spike').

First, save the k-space data of slice 80 under a new name (so we can keep the original data intact),

kanatslice80_bad=kanatslice80;

and then change the magnitude value at a single point of that slice:

kanatslice80_bad(129,110)=90000000;

This is a point just to the left of the center of k-space (x-y coordinates 129,110), and you've just artificially set the magnitude of its spatial frequency to be very large (i.e. the 2D inverse Fourier transform will now think that this particular spatial frequency is strongly represented in the signal). Let's plot the signal in this line of k-space again, just like you did above:

figure

plot(abs(kanatslice80_bad(129,:)));

Note how the signal intensity of point 110 is now very large (even larger than at the center of k-space); essentially, we have just manipulated k-space: the signal of the spatial frequency at k-space point [129,110] is now very strong!

Now, use the inverse 2D Fourier transform of this manipulated k-space to see the resulting image:

anatslice80_bad=ifft2(fftshift(kanatslice80_bad));

figure

imagesc(abs(anatslice80_bad));

colormap('gray')

axis('image')

Q1: What do you observe in the new image?

Plot the center line in kanatslice80_bad and compare it to the original plot of this line in kanatslide80. You just increased the value (signal intensity) at a frequency directly to one side of the center of k-space.

Now, choose different points in k-space (and report the coordinates you use) to create the following patterns:

Q2: Diagonal stripes in this direction:  ////

Q3: Diagonal stripes in the opposite direction: \\\\

Q4: High frequency horizontal stripes.

4. Effect of using a subset of k-space

We will now look at what happens if you 'eliminate' parts of k-space by setting them to zero.

First, we will set the outer edges of k-space to zero.

kanatslice80_SFlow = zeros(256,256);

kanatslice80_SFlow(115:144,115:144) = kanatslice80(115:144,115:144);

figure

imagesc(abs(20*log10(kanatslice80_SFlow)));

colormap('gray')

axis('image')

You can see that we are now just using the center of k-space; let's use this k-space data and see what it looks like in the image domain:

anatslice80_SFlow = ifft2(fftshift(kanatslice80_SFlow));

figure

imagesc(abs(anatslice80_SFlow));

colormap('gray')

axis('image')

Q5: How does this new image compare to the original image?

Let's now create an image by setting the interior of k-space to zero.

kanatslice80_SFhigh = kanatslice80;

kanatslice80_SFhigh(115:144,115:144)=0;

figure

imagesc(abs(20*log10(kanatslice80_SFhigh)));

colormap('gray')

axis('image')

anatslice80_SFhigh = ifft2(fftshift(kanatslice80_SFhigh));

figure

imagesc(abs(anatslice80_SFhigh));

colormap('gray')

axis('image')

Q6: How does this new image compare to the original image?

Names:____________________

Summary of Exercises

Q1: What do you observe in the new image?

Report the coordinates used to create:

Q2: Diagonal stripes in this direction:  ////

Q3: Diagonal stripes in the opposite direction: \\\\

Q4: High frequency horizontal stripes.

Q5: How does this new image compare to the original image?

Q6: How does this new image compare to the original image?

# Week 3 Exercises: Understanding Contrast

This week's exercises will illustrate the principle of contrast and how it relates to MRI pulse sequences.

The text (Chapter 5) and lecture discussed the idea that MR images can be acquired with different forms of contrast.

Anatomical images (T1.nii, T2.nii) for this part of the lab can be found on Canvas, under Files/Week 3/

The datasets are two high-resolution anatomical volumes. The T1 and T2 scans were acquired axially, and are sensitive to the contrasts their names denote.

# Exercise 1: Loading and displaying the data

In SPM, select Check Reg in the SPM Menu window, and then select both datasets (T1.nii, T2.nii). This will display both scans in the Graphics window. If you right click on any of them, a whole lot of information pops up, including the file name, voxel coordinates, and signal intensity (Y) at that voxel.

Q1: Look at the two different axial images (T1, T2). What is similar about the datasets?

Q2. What is different about the datasets?

# Exercise 2: Quantifying the datasets

## 2.1 Recording values of each tissue type

In the Graphics window, select a voxel in the white matter of the T1 scan; since the two images are in register, the same voxel will be selected in the other scan (T2). By selecting different voxels in white matter, roughly estimate the range of intensity values for the white matter tissue type in each image (by right-clicking and reading out the Y value), and enter that range in the chart. Repeat for the other tissue types listed in the chart for Q3.

Q3 should be filled out on the answer key at the end of the laboratory.

| Image | White Matter | Gray Matter | CSF | Skull | Air |
| --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |
| T2 |  |  |  |  |  |

# Review: Basic properties of MRI pulse sequences

# The T1 scan was acquired using a pulse sequence with the following key parameters:

TR: 500ms, TE: 25ms

# Q4. What does the abbreviation TR stand for? It is the time between two events in a pulse sequence: what are those two events?

# Q5. Why is TR typically expressed in seconds? What time constant is most associated with TR?

# Q6. What does the abbreviation TE stand for? It is the time between two events in a pulse sequence: what are those two events?

# Q7. Why is TE typically expressed in milliseconds? What time constant is most associated with TE?

# Summary of Exercises

Q1: Look at the two different axial images (T1, T2). What is similar about the datasets?

Q2: What is different about the datasets?

Q3 should be filled out on this answer key:

| Image | White Matter | Gray Matter | CSF | Skull | Air |
| --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |
| T2 |  |  |  |  |  |

# Q4. What does the abbreviation TR stand for? It is the time between two events in a pulse sequence: what are those two events?

# Q5. Why is TR typically expressed in seconds? What time constant is most associated with TR?

# Q6. What does the abbreviation TE stand for? It is the time between two events in a pulse sequence: what are those two events?

# Q7. Why is TE typically expressed in milliseconds? What time constant is most associated with TE?