Names:

Week 5, Lab 3: Preprocessing

Once you have acquired your fMRI data, there are a few so-called preprocessing routines that you need to run on your data before you can attempt any kind of statistical analyses.

The functional data that you get when you download it from BIAC will already be in image space (remember that data are acquired in k-space).

While it may be convenient to eventually set up a preprocessing pipeline (e.g. a script) that automatically runs through all the preprocessing steps for each participant, it is important to check the results at least for every participant, if not each individual step for each individual. In this lab, you will go through the main preprocessing steps using the GUI.

The main preprocessing steps are realignment of the sometimes hundreds of volumes acquired as part of the fMRI study, slice timing correction to correct for the difference in time with which each slice in a volume was acquired, co-registration with a high-resolution anatomical scan from the same participant, segmentation of the anatomical scan to derive parameters for normalization into a standard stereotactic space, and finally spatial smoothing to increase the signal-to-noise ratio for subsequent statistical testing.

Side note:

There are multiple different ways of preprocessing your data, from the various routines that you might run, their order, and the tweaking of various parameters within the routines. There is no one-fits-all approach, and different researchers have different preferences; what is important, though, is that you describe very clearly - once you have acquired your own data and are writing up the results in your manuscript - which steps you took and it what order. The idea is that you describe your analysis such that another researcher can replicate your analysis based on your description.

The current data set was acquired using the GE 3 T scanner at BIAC and consists of 160 volumes of 30 slices each, acquired with an EPI T2*-weighted sequence. Important scanning parameters are TR: 2 s; TE (TA): 27 ms; flip angle: 77°; field of view (FOV): 240 × 240, voxel size 3.75 x 3.75 x 4 mm. The slices were acquired in the following ascending interleaved fashion [1 3 5 ... 29  2 4 6 ... 30].

The task the participant was performing is the same as in the dataset we looked at in the first lab: alternating between squeezing left and right hands with rest periods in-between.

# Realignment & Unwarping

The Realign (Estimate & Reslice) routine realigns all the volumes using a least squares approach and a 6 parameter (rigid body) spatial transformation. The first image specified by the user is used as a reference to which all subsequent scans are realigned (this is usually the first scan of the entire scanning session, but can be any other scan). Once it has estimated the movement parameters of each volume (Estimate) with respect to the reference volume, it subsequently applies these spatial transformations to each volume (Reslice).

To get started, download the data from Canvas (Files/Week 5) and unzip it (this will create a folder called Lab_03) into your NEUROSCI382 folder. The zip file contains a Func folder with two files in it (two runs of that participant) and an Anat folder with the anatomical scan of that participant.

Start Matlab, go to your Lab_03 folder, and then start SPM by typing spm fmri at the prompt in the Command window. Select the Realign button and, from the drop-down menu, select the third option Realign (Est & Res). This will open up the Batch Editor GUI.

Select a New Session and click ‘Specify’. In the Lab_03 Func folder, type   bia5_20105_006_01.*  in the field next to Reset button, and then type 1:160 in the field below and hit Enter. This will show all 160 volumes. Click on the first one, then scroll down and - while keeping the Shift key pressed - select the last one; this will move all volumes into the bottom field. Click Done. Then create a New Session, and repeat the same steps for the second run (bia5_20105_007_01.*).

You can leave all the other parameters as they are.

In the Batch Editor, the triangle pointing to the right should now be green, which tells you that you've provided all the necessary information so that you can now run the Realign preprocessing routine.

Before running it, however, go to File  Save Batch and save this Realignment part of the preprocessing steps as something like 'Batch_01Realign'. This saves the realignment procedure with all the parameters that you entered so that you can go back to it at a later stage.

Now run the Realign routine by pressing the green triangle. The final step is the Reslicing procedure in which all images are realigned based on the determined motion parameters (6 parameter spatial transformation). Both Realign and Reslicing procedures are run consecutively and you'll see Done printed in the Matlab Command window once this preprocessing step is complete.

- Make a note of what files have been created by running this preprocessing routine (see the table at the end).

Open the file spm_2018Sep25.ps. What does the plot in the Graphics figure show?

# Slice timing correction

The slices of each volume were acquired in an interleaved fashion, so we need to correct for the resulting timing differences. Select the Slice timing button in the upper left window. This will open up the Batch Editor again.

Click on “Data” and choose a New Session by clicking on the blue-highlighted text and then select the 160 scans of the run in the following way: Select the newly created field Session and then click Specify...; this will open up a new window for you to select the scans. In the Func folder, type   ^r.*   in the field next to the Reset button, and then type 1:160 in the field below and hit Enter. This will show the 160 volumes each of both runs. Click on the first one, then scroll down to the last volume of the first session (i.e., all the filenames that end with 006_01) and - while keeping the Shift key pressed - select it; this will move all volumes into the bottom field. Click Done.

Repeat these steps for the second run (files ending with 007_01).

Back in the Batch Editor, enter the Number of Slices, the TR (in seconds) and TA by clicking on ‘Specify’. (see the formula provided in the bottom of the SPM window). Then enter the Slice order by typing [1:2:30 2:2:30]. As Reference Slice select the first slice, i.e. type 1.

Go to File  Save Batch and save the Slice Timing correction part of the preprocessing steps as something like Batch_02SliceTiming.

The sideways triangle should now be green, which tells you that you've specified all the necessary information and that you can now run the Slice Timing preprocessing routine.

- Make a note of what files have been created by running this preprocessing routine (see the table at the end).

# Co-registration

EPI BOLD images have rather low spatial resolution, yet we want to know where in the brain the HDR changes systematically with the experimental manipulation. One way is to superimpose the statistical results from the BOLD images on the high-resolution anatomical scan of that participant. In order to do this, we first need to co-register the functional and anatomical data, i.e. make sure that they line up as well as possible. To do this, click on Coregister and, from the dropdown menu, select the first option Coregister (Estimate) to open this in the Batch Editor GUI. As Fixed Image (or Reference Image) select the mean functional volume that was created during the realignment (prefixed with  mean*), and as Moved Image (or Source Image) select the anatomical scan  bia5_20105_003.nii (in the Anat directory).

Save this as Batch03_Coregister and press RUN.

This co-registers the anatomical scan to the mean functional scan.

- Make a note of what files have been created by running this preprocessing routine (see the table at the end).

To check how well in register the images are, go to Check Reg in the upper left window (the one with green background) and select the anatomical and the mean functional scans.

# Segmentation

The Segmentation preprocessing procedure segments the brain into 3 different tissue types (gray matter, white matter, CSF), based on normalized (template) versions of these tissue types. SPM will also write a spatial normalization deformation field in the Anat directory. This data will be used in the next step to normalize the data.

Select Segment to open the Batch Editor GUI and under Volumes choose the co-registered anatomical scan (it still has the same name as before). Under Save Bias Corrected just below, choose Save INU Corrected (or Save Bias Corrected) instead of the default Save Nothing. Then scroll down and choose, for the last parameter (Deformation Fields) the option Forward, instead of None.

Save this as Batch_04Segment and Run.

- Make a note of what files have been created by running this preprocessing routine (see the table at the end).

# Normalization

Every brain is different; even identical twins have different cortical gyrations! In order to make comparisons between participants and in order to be able to make generalizations with respect to the general population, we need to transform the fMRI data into a common stereotactic reference space. This ensures that a certain brain region, e.g. V1, is (roughly) at the same location in this reference space for every participant, allowing comparison across participants and also across studies.

SPM uses the so-called MNI space, which is based on anatomical scans of hundreds of neuro-typical, right-handed individuals conducted at the Montreal Neurological Institute.

Click on Normalize and choose the second option Normalize (Write) to open up the Batch Editor GUI; under Data, choose New: Subject, and for Deformation Field select the y_*.*  file in the Anat folder that was created during the segmentation procedure; this contains the normalization parameters that were estimated during the segmentation procedure (and which will now be applied to the images we want to normalize). Select the Images to Write field and go into the Func folder. Select all the realigned functional images for both runs (type arbia* in the 'Filter' box, and enter 1:160 in the 'Frames' box; this time you don't need to treat them as separate runs and can just load all 320 of them).

Before running this step, save it as Batch_05Normalize.

Once this has finished, repeat the normalization for the structural scan: repeat the steps as above, but instead of choosing all functional images as the images to align, this time choose the mbia5*.* anatomical scan (the m*.* scan is a slightly brighter version of the original structural scan). Also, under Voxel sizes, change from [2 2 2] to [1 1 1] isotropic voxels, since we don't want to lose the high structural information of the anatomical scan.

Before running this step, save it as Batch_05NormalizeStructural.

- Make a note of what functional and anatomical files have been created by running this preprocessing routine (see the table at the end).

Now check the results. For example, choose Check Reg and select a random functional normalized volume (starting with 'w*', in the Func folder) and in addition click on the Prev field at the top of the pop-up window and go to the spm12 program folder and then navigate to the subfolder named 'canonical'. Select the   avg305T1.nii   anatomical scan (this is the average normalized brain of 305 people in MNI space) and press Done. Note how well they align.

- Can you think of why the avg305T1.nii scan is so blurry?

The  'single_subject_T1.nii'  file in the 'canonical' folder represents an anatomical T1-weighted scan of an individual brain out of the MNI database that matches the stereotactic reference MNI space most closely. Compare the single_subject_T1.nii anatomical scan with the normalized anatomical you just created (again via Check Reg).

# Smoothing

The final preprocessing step is to smooth the data by applying a spatial smoothing kernel to each voxel. The size of the smoothing kernel, expressed in Full Width Half Maximum (FWHM), needs to be commensurate with the data that you are looking at. A general rule of thumb is that FWHM should be about twice the voxel size. However, anatomical considerations are also important. For example, if you are interested in a very small structure such as the inferior colliculus (an important subcortical station in the auditory pathway), you may want to apply a smoothing kernel of FWHM = 4 mm (or even not smooth at all, depending on your voxel size). However, for most cortical data a FWHM of 6-8 mm is recommended.

Select Smooth to open the Batch Editor GUI and as Images to Smooth select all normalized functional scans (i.e. those starting with w*), again for both runs (total 320). Change FWHM to 6 6 6.

Save this as Batch_06Smoothing, and press RUN.

- Make a note of what files have been created by running this preprocessing routine (see the table at the end).

Using Check Reg, compare the normalized and the normalized & smoothed functional scans. How are they different?

1) Realign & Unwarp

2) Slice timing

3) Co-registration

4) Segment

5) Normalization

6) Smoothing